The NIDAQ data is collected at 20000 Hz, and is the most reliable time stamp that everything else should align themselves to.

NIDAQ data is stored in the .mat file that is being read.

row1 = Data[0, :]  # 时间(秒) / time in seconds

row2 = Data[1, :]  # AI 0, WBA Left 左翼拍幅

row3 = Data[2, :]  # AI 1, WBA Right 右翼拍幅

row4 = Data[3, :]  # AI 2, WBF 翼拍频率

row5 = Data[4, :]  # AI 3, DAQ1, Arena x position 竞技场 x 位置

row6 = Data[5, :]  # AI 4, EMG 或 Hutchens 触发信号

row7 = Data[6, :]  # AI 5, LED 驱动触发信号

row8 = Data[7, :]  # AI 6, Right hutchens 右侧 Hutchens

row9 = Data[8, :]  # AI 7, Basler 相机触发信号

My task is to align the NIDAQ data, a data calculated from Basler camera video, and a data calculated from Phantom camera video. The Basler camera is triggered by signal in row 9, and the Phantom camera is triggered by signal in row 6. Those signals are all TTL signals, ~0V meaning not triggered, a signal larger than (I think) 1.5V means it’s being triggered, and both cameras are triggered on the rising edge.

However, the Phantom camera is only recorded during a time range, set by the rec_on_time_s and rec_off_time_s. Therefore, I only need data collected in these time ranges, for both the NIDAQ, Basler and Phantom data.

First step I want to visualize the trigger signals for Phantom data (row 6). That is code block 1. This is only visualization and for inspection.

Second step I want to detect all rising edges in the Phantom trigger signal (row 6), then assign a timestamp to all these triggers. But only the rising edges within rec_on_time and rec_off_time. The frame count start from 1 for the first frame within the rec_on_time and rec_off_time, but the timestamp should not – it should be in the common timeline like in the NIDAQ row 1. Save that data (frame_number as first column, time as second column), plus the wingbeat frequency data, which is 100*values in row 4, in the same time frame as the third column, in a csv file named the same as the mat file except with a suffix _phantom_trigger_timedstamp.csv. This is the second code block.

Then I will manually add in the data called normalized intensity as the fourth column in the phantom_trigger_timestamp.csv. This is the normalized intensity for spiracles, but it need to be rescaled, where the average of the minimum 5 values is 0, and average of the highest 5 values is 1. Save the data in the fifth column called rescaled intensity, and plot the rescaled intensity against time. This is the third code block.

Then I want you to process the Basler trigger signals. Similarly visualize the signal first. This is the fourth code block.

Then I want you to process the Basler video data. The Basler trigger signals need to be saved from frame 1, with time in the common timeline as the NIDAQ data. Then you should read in another csv with the same base name as the .mat file, but with extension _TopCamera.csv. This data has three important rows, Frame, which is aligned with the Basler trigger signal (thus aligned with the common timeline), and LeftWing and RightWing (just search for columns containing these keywords, don’t do exact match because the name might have spaces etc), two columns storing the data for left and right wing amplitude. You should calculate the value wingbeat_amplitude, which equals to LeftWing + RightWing +3.1416. Then apply the rec_on_time and rec_off_time to the data, and only save the data within those time. Save it to the csv file with basename_basler_trigger_timestamp.csv. First column frame_number_basler (starting from 1, which is the first frame within rec_on_time and rec_off_time), second column time (which is still in the timeline of the NIDAQ data), third column wingbeat_amplitude. This is the fifth code block.

Finally, I want you to combine the basler_trigger_timestamp.csv and phantom_trigger_timestamp.csv. They are on the same timeline at this point I think.  Resample the data at 20000 Hz. Plot them on the same timeline with 20000 Hz sampling frequency (do tell me what resampling method you’re using). Only save the data time_s (column 1, but this time recalculate time such that the starting time is 0), rescaled_intensity (column 2), wingbeat_amplitude (col 3) and wingbeat_frequency (col 4). But do calculate a fourth value, inferred_flight_power (col 5), which = P_pro* + P_ind*, where P_pro* = C_pro · Φ³ · n³ (profile power, from wing drag), P_ind* = C_ind · Φ^(−1/2) (induced power, from generating lift). C_pro = 1.44 × 10⁻⁷ m², C_ind = 10.5 m²s⁻³. Save the data as basename_combined.csv. This is the 6th code block. Visualize the data, specifically plot the rescaled intensity in subplot 1, wingbeat amplitude in subplot 2, wingbeat frequency in subplot 3, inferred power in subplot 4. This is code block 6.

In the last code block, I want you to calculate the pearson correlation coefficient for rescaled intensity against wingbeat frequency, amplitude, and inferred power. I want you to calculate the correlation coefficient from (recalculated time) time 10-30s, and time 50-70s. Save those values in a new csv file correlation_score.csv. First column is basename, second column pearson_corr_score_10_30s, third column pearson_corr_score_50_70s. I also want you to calculate the area under the curve for 10-30s and 50-70s. save it as fourth column AUC_10_30s and fifth column AUC_50_70s. Because I will keep processing each .mat file, each time when you write new data in this csv, append it to the next row. This is the code block 7.

That’s it. Please be sure to do as I said, and if anything is unclear, ask me before implementation. Please do one block at a time, ask me to try the block, and then proceed to the next when I say yes.